This document is a Microsoft Word version of the of a web page on F2D.dk

If you have proposals for additions or changes you are welcome to use this document for feedback.

Corrections to spelling mistakes are also welcome.

To make it easier for me to track changes please follow these recommendations:

Do not delete text from the document but use strikethrough. (example)

If you add text please use a colour font.  (example)

You may also use the markup functions under “Review” (Microsoft Word)

You are welcome to fill in your name and e-mail address in the top of this document.

It is my plan to send out early notifications on updates to supporters and helpers.

Name:

E-mail:

Please e-mail this document with changes to: henning@forbech.dk

Thanks in advance

/Henning Forbech

Back to F2D.dk - Streamer

Experimental streamer for F2D
Design version 1.0
First published: 13/9 2012 - Last update: 29/12 2012 - By Henning Forbech 

This page is under construction!

| Intro: (some general info on the experimental streamer here) |  |  |
| --- | --- | --- |
| Steel ring: The experimental streamers come with an integrated steel ring. This ring is a part of the streamer. The ring will make it easy to take off the streamer and to hook it on to another model. No need to tie knots or loops on the streamer. No fancy streamer hooks with sharp edges to weaken the loop on the streamer string. Short string: The string is very short. Only long enough to keep the paper part free of the model. The reason for the short sting is to prevent pilots from giving away the streamer. With a very short streamer string it will be very dangerous and probably not possible to fly you model in front of you opponent to make him take all streamer in one go. The material for the string on this experimental streamer is similar to the streamers at the world champs in Bulgaria. Maybe the string should be a little stronger but if the string is too strong it will damage models that actually hit the string. No knots: The connections on the streamer string are made by glue and heat shrink plastic tube. At the ring the string is wrapped around the metal and glued to itself with contact glue. At the streamer head the sting forms a loop and is glued to itself. The glued parts are covered by a piece heat shrink plastic tube. This design will probably have lower tendency to damage the model when a streamer is pulled over a wing than the current design with a big and heavy knot. Streamer head: The new streamer head with sloping shoulders should minimize the chance of a streamer hanging on the head. This will also lower the chance of parts of the opponent's streamer hanging on the streamer. The streamer head is made of semi-transparent tape with glass fibre reinforcements. This will let the most of the head have the same colour as the paper part of the streamer. Double paper streamer: The streamer consists of two paper streamers. A very long streamer and a short "extra" streamer. The two streamers are connected in a common head and will act as one streamer when flying. If the long streamer breaks at the head the short streamer will act as a spare streamer. This will leave some paper for the opponent to trace and he will still have a chance to score a cut. Cuts made to the double streamer will be seen as one cut and should also only count as a single cut. Long paper streamer: The long paper streamer is 4.5 meter. The paper is biodegradable forestytape. Same material as used for the streamers at the world champs in Bulgaria. The paper is 25 mm wide and is cut with a zig zak scissors. The zig zak shape indicate that the streamer have not been use. Short paper streamer: The short paper streamer is 0.5 meter. The star shaped hole at the end of the short paper streamer is a side effect from the manufacturing on streamer machine |  |  |
| The head of the experimental streamer. Sloping shoulders and no knot. Transparent tape gives the head the same color as the paper This video demonstrate how to make the knot The new experimental streamer knot and a classic streamer knot A piece of paper hanging on the knot of a classic streamer |  |  |
| The steel ring: The raw steel ring is made from zinc plated mild steel wire. The wire is ø 1.2 mm and the inside diameter of the ring is 20 mm |  |  |
| If the twisted wire is cut with a diagonal cutter there will be some sharp burrs at the end. These burrs might cut the string inside the shrink tube. A scissor-style cutter (cable cutter) was used to cut the wire for the experimental streamers. Scissor-style cutter (cable cutter) with blue handle and diagonal cutter with red handle The cable cutter have been modified for the wire size The difference between wires cut with a scissor-style cutter a diagonal cutter These sharp burrs might cut the string Wire cut with a modified cable cutter. No burrs - no problem. The string is soaked with contact glue before applying the shrink tube A piece of shrink tube are put over the connection The heat shrink is shrieked with a heat blower. This could also be done in an oven |  |  |
| Packaging: The paper wrap around the streamer is to prevent the streamer from unfolding before it is handled to the pilot. Classic streamers often come with a rubber band but this rubber band may cut into the paper when it is pulled off. The wrapping can be marked with the manufacturers name, product number, batch number, etc. Maybe the wrapping can also be used for sponsor add. This streamer is sponsored by … |  |  |
| Test in competition: The experimental streamer was tested first time at the Penguin Club in Denmark The experimental streamer was used in the final at the Danish Championship 2012 |  |  |
| First test in USA Design version 1.0 of the experimental streamer have been tested at a F2D competition in Halifax, Massachusetts. Mark Rudners report on the experiment can be found in his F2D News - November 2012 2012 Fall Finale: Mark Rudner, Matt Stas and Brian Stas (Photo from New England Combat News) |  | New England Combat News A lot of nice photos from the competition At Mark Rudner's F2D News in the F2D News - November 2012 you will find more information on the test at this competition Bob and Andy Mears tried a streamer after the 2012 WC's Combat-I |
| Next step: The experience from the first tests lead to some changes to the design. The experimental streamer will now be upgraded to version 2.0 and the next test will be at a competition in Russia. Follow the development and test of the Experimental Streamer, Version 2.0 |  |  |
| Your comments: Your are welcome to comment on the experimental streamer, please use: Combat-I, Barton or FAS Permalink: http://www.f2d.dk/equipment/streamer/streamer-10.htm - Public domain - Spell checking doc |  |  |